- USB cable is connected to the device and the device is turned on with the built-in battery switch

- 2.Select the driver and install the driver

- 3. The program is installed successfully

- 4.Open sscom, follow the serial number 1-2-3-4, and finally issue the command

- 5.Note. If you need to send a query parameter command, you need to open the log mode to query the parameters

- AT%TEST=*1234,280*1# Open log print
AT%TEST=SMS[][PARAM#]

- AT%TEST=*1234,280*0# close log printing



6.Assuming that you need to set multiple parameters, such as setting ip, port and apn, you can open the loop command mode of the device

- Notes

- 1. If the correct port is selected and the device does not respond after issuing the command

- 2. It may be that the red power supply line is connected to the USB motherboard, which causes the command to fail to be issued successfully

- 3. Solution: Disassemble the USB interface, cut off the red USB line directly, and re-issue the command to solve the problem of the command not being able to respond.